< How to - USB Driver installation >

For the first time, follow the instructions from 1 to 13. This procedure is required for initial use for once. 
1. Download and install the Flash_tools program on your PC (important) 
2. AC power Off in the rear panel. (This is to make Cold Off mode)
3. Connect your PC with PVR by USB Cable 
4. AC Power On.
5. Press and keep hold the Power button in RCU more than 2seconds until you see “USB UPGRADE” in the front panel or PENGUIN in TV. 
6. PC detects new hardware device (important) 
7. Driver Installation Wizard Appears 
8. Select ‘install from a list or specific location(advanced)’ and click ‘Next’. 
9. Select ‘Include this location in the search’ and click ‘Browse’
10. Designate the folder at C:/Program files/Flash_tools) and click ‘Next’
11. Wizard will search the driver file.  

12. Select ‘USB FOR VESTA – c;/windows/inf/vestausb.inf’ and click ‘Next’
12.1. If it asks system file in some case, Browse at c:/program files/Flash_tools/vestausb and click ‘OK’
13. Click ‘Finish’ when it is done. 
Important) The order of this procedure is very important; please do follow the instruction step by step. 
When Driver is installed successfully, it is ready to download the latest image to the PVR (USB download instructions) 

< How to - Downloading new Image >

1. Your PVR and PC have to be connected by USB cable. 
2. The PVR has to be in ‘USB upgrade mode.  
    * For the ‘USB upgrade’ mode, refer to above Driver installation procedure from 1 to 5.

    * Tips to make USB upgrade mode without AC Power ON/OFF 

        - If you are watching TV now, press Power button in RCU more than 2 seconds. This              status is same as AC OFF/ON which is so called ‘Cold off’. And then, Press and keep hold the Power button in RCU more than 2seconds until you see “USB UPGRADE” in the front panel or PENGUIN in TV.
3. Run the Flash Tools 
4. Select ‘USB’ and check the “USB connected” in the Message box. 
5. Designate the image that you want to download to the PVR by pressing FILE 
6. Click ‘Down’ and Wait for "DONE" on the front display
    * The progress is Downloading -> Erasing -> Writing.

    * Downloading time : depends on what image. But in case of ‘usb_all_noboot.img(about 8MB), it takes about 6 minutes.

7. Press Power button to reboot. your PVR will be restarted 
Important) To enter “USB MODE” : Turn the PVR into standby mode(press the power button more than two seconds) then press and hold power button again until you see “USB” on the display of the PVR.
* Tips :

- Standby mode : There are 2 kinds of Standby mode in this PVR, Cold Off and Warm Off.  While watching TV, if you press power button on RCU or Front panel just one touch, your PVR is in Warm Off.  If you press the power button more than 2 seconds, your PVR will be in Cold Off.  To precede the USB upgrade, make sure that your PVR is in Cold Off first.
